Below is a Functional Requirements Specification (FRS) format for the Niagara 4/5 Lenel OnGuard OpenAccess Driver. It keeps the first release primarily read-only, with OnGuard remaining the authoritative system and Niagara consuming approved information through OpenAccess.

Functional Requirements – Lenel OnGuard OpenAccess Niagara Driver

1. Purpose

The Lenel OnGuard OpenAccess Niagara Driver shall integrate Niagara 4/5 with Lenel S2 OnGuard through the OpenAccess platform.

The driver shall provide secure connectivity, authentication, discovery, device-status monitoring, event subscription, alarm generation, history, and visualization of supported OnGuard panels/controllers, readers, logical doors, and security events.

OnGuard shall remain the system of record for identities, credentials, permissions, schedules, alarms, and access-control policies. Niagara shall consume approved information for visualization, alarming, history, analytics, occupancy, and operational purposes.

2. High-Level Functional Architecture

LENEL S2 ONGUARD

[Authoritative System]

│

┌───────────┴────────────┐

│                        │

OpenAccess REST          Event Services

│                        │

└───────────┬────────────┘

│

HTTPS

│

▼

NIAGARA LENEL DRIVER

┌────────────────────────────────┐

│ Authentication / Session       │

│ API Version Management         │

│ Discovery                      │

│ Status Management              │

│ Event Subscription             │

│ Event Filtering                │

│ Data Normalization             │

│ Alarm / History Management     │

│ Diagnostics / Recovery         │

└───────────────┬────────────────┘

│

▼

NIAGARA OBJECT MODEL

│

┌───────────┼─────────────┐

▼           ▼             ▼

Panels      Readers     Virtual Doors

│           │             │

└───────────┼─────────────┘

▼

Points / Events

Alarms / History

│

▼

Niagara Supervisor/HMI

3. Connectivity Requirements

FR-CON-001 – OpenAccess Connectivity

The driver shall establish communication with the Lenel OnGuard OpenAccess service using HTTPS.

FR-CON-002 – Server Configuration

The driver shall provide configuration for the OnGuard/OpenAccess server information required to establish connectivity.

FR-CON-003 – TLS

All production communication between Niagara and OpenAccess shall use secure TLS communication.

FR-CON-004 – Connection Status

The driver shall expose the current connection status as a Niagara property.

Recommended states:

Disconnected

Connecting

Connected

AuthenticationFailed

CommunicationFailure

FR-CON-005 – Connection Recovery

The driver shall automatically attempt reconnection following temporary OpenAccess or network communication failures.

FR-CON-006 – Driver Health

The driver shall expose driver health information including:

Connection Status

Last Successful Communication

Last Communication Failure

Authentication Status

Event Subscription Status

Last Event Received

Last Synchronization

4. Authentication Requirements

FR-AUTH-001 – Integration Account

The driver shall authenticate to OnGuard using a dedicated integration account.

FR-AUTH-002 – Authentication Parameters

The authentication configuration shall support:

Username

Password

Optional Directory ID

Application ID

OnGuard Server

The source records username/password as required, Directory ID as optional for the internal-directory flow, and Application ID as required for authentication.

FR-AUTH-003 – Secure Credential Storage

The integration password shall be stored using Niagara's secure credential/password mechanism and shall not be exposed as plain text.

FR-AUTH-004 – Session Token Management

The driver shall provide a Session Manager responsible for:

Login

Logout

Token caching

Token expiration handling

Token renewal

Reauthentication

These responsibilities are already identified in the driver design.

FR-AUTH-005 – Token Expiration

The driver shall obtain and maintain the session-token expiration information returned by OnGuard rather than relying solely on a hard-coded timeout.

FR-AUTH-006 – Reauthentication

When the current session becomes invalid or expires, the driver shall attempt controlled reauthentication using the configured integration credentials.

FR-AUTH-007 – Authentication Failure

The driver shall distinguish, where supported by OpenAccess, between:

Invalid credentials

Expired password

Expired session

Locked account

Communication failure

Authentication service failure

FR-AUTH-008 – Event Recovery after Reauthentication

Following reauthentication, the driver shall verify the OpenAccess event subscription and recreate/reconnect it when required.

5. Password Lifecycle Requirements

FR-PWD-001 – Password Expiration Monitoring

The driver shall retrieve and expose password-expiration information available through supported OpenAccess APIs.

FR-PWD-002 – Password Warning

The driver shall provide a Niagara-visible warning when the integration account password is approaching expiration or has expired.

FR-PWD-003 – Password and Token Separation

Password expiration and session-token expiration shall be maintained as separate properties and shall not be treated as the same lifecycle.

The current design specifically requires these values to be handled independently.

FR-PWD-004 – Password Renewal

Password reset/renewal through Niagara shall not be supported in V1.

When the password is renewed in OnGuard, an authorized Niagara user shall be able to update the corresponding Niagara integration credential.

This matches the current first-release decision.

6. API Version Requirements

FR-API-001 – OnGuard Version Detection

The driver shall retrieve available OnGuard/version information through the supported version API where available.

FR-API-002 – API Version Resolution

The driver shall determine the appropriate supported API version for each required OpenAccess operation.

FR-API-003 – API Compatibility

The driver shall not assume that a single OpenAccess API version applies to every API operation.

Your design notes specifically identify that OpenAccess APIs have individual versions and that compatibility must therefore be evaluated API-by-API.

FR-API-004 – Additional Response Attributes

The driver should tolerate additional non-breaking attributes returned by newer supported OpenAccess API versions.

7. Panel/Controller Discovery Requirements

FR-PNL-001 – Panel Discovery

The driver shall discover supported OnGuard access panels/controllers using the approved OpenAccess API.

FR-PNL-002 – Panel Selection

Niagara users shall be able to select discovered panels to be added to the Niagara station.

FR-PNL-003 – Panel Identity

Each panel shall retain its stable OnGuard identifier for event and status correlation.

FR-PNL-004 – Panel Properties

The driver shall expose supported panel properties as read-only Niagara properties, including where available:

Panel ID

Name

Panel Type

Firmware/Version

Online Status

Health

Last Successful Read

Last Status Change

These properties align with the current controller model.

FR-PNL-005 – Panel Online Status

The driver shall expose the current panel communication state as:

ONLINE

OFFLINE

UNKNOWN

FR-PNL-006 – Communication Lost

Upon receiving an approved Communication Lost event associated with a configured panel, the driver shall:

Set panel status to OFFLINE

Raise/configure the applicable Niagara alarm

Record the event/history

Update last-status-change time

FR-PNL-007 – Communication Restored

Upon receiving the corresponding Communication Restored event, the driver shall:

Set panel status to ONLINE

Normalize the active communication alarm

Record the restoration in history

Update last-status-change time

This behavior follows the current panel-status design.

8. Reader Discovery Requirements

FR-RDR-001 – Reader Discovery

The driver shall discover readers associated with selected OnGuard panels.

FR-RDR-002 – Selective Addition

The user shall be able to select which discovered readers are added to Niagara.

FR-RDR-003 – Reader Association

Each reader shall maintain an association with its parent panel/controller.

FR-RDR-004 – Reader Identification

The driver shall maintain the appropriate OnGuard reader identifier required for API and event correlation.

FR-RDR-005 – Reader Properties

Where provided by the supported OpenAccess API, the following shall be exposed as read-only Niagara properties:

Reader ID

Controller/Panel ID

Reader Name

Reader Mode

Offline Mode

Strike Time

Entry Area

Exit Area

Online/Hardware Status

Last Event

Last Event Time

The existing model identifies reader ID, controller ID, name, online status, mode, strike time, entry area, and exit area.

FR-RDR-006 – Initial Reader Status

Where supported, the driver shall retrieve the initial reader hardware status during discovery or synchronization.

FR-RDR-007 – Unknown Reader Status

The driver shall not infer reader online/offline state from unrelated OpenAccess properties.

If reliable reader status cannot be determined, Niagara shall represent it as UNKNOWN.

This is consistent with the current investigation into reader hardware status.

FR-RDR-008 – Reader Status Events

Approved reader communication/status events shall update the corresponding Niagara reader object.

9. Door Requirements

FR-DOOR-001 – Logical Door Representation

Where OpenAccess does not expose a separate physical door object suitable for discovery, the Niagara driver shall support a logical/virtual door representation associated with the relevant reader configuration.

The current API investigation supports this logical grouping approach.

FR-DOOR-002 – Reader Association

A logical door shall support association with the applicable:

Entry Reader

Exit Reader, where available

Parent Panel

FR-DOOR-003 – No Reader Duplication

A reader shall exist as a single Niagara component. Logical doors shall reference reader components rather than creating duplicate reader components.

FR-DOOR-004 – Door State

Where supported by OnGuard data/events, Niagara shall expose:

Open

Closed

Forced

Held

Unknown

FR-DOOR-005 – State Separation

Where sufficient information is available, Niagara should maintain physical position and security conditions independently:

doorPosition = OPEN / CLOSED / UNKNOWN

forced = TRUE / FALSE

held = TRUE / FALSE

A derived door state may additionally be exposed for visualization.

FR-DOOR-006 – Door Forced Alarm

An approved Door Forced event shall update the logical door state and generate the configured Niagara alarm.

FR-DOOR-007 – Door Held Alarm

An approved Door Held event shall update the logical door state and generate the configured Niagara alarm.

FR-DOOR-008 – Door Restoration

Corresponding restoration events shall normalize the applicable Niagara alarm and update the current door state.

10. Event Subscription Requirements

FR-EVT-001 – Event Subscription

The driver shall establish an OpenAccess event subscription for approved OnGuard event categories.

FR-EVT-002 – Supported Event Categories

V1 event categories may include the approved subset of:

Access Granted

Access Denied

Communication Lost

Communication Restored

Reader Status Events

Door Open

Door Closed

Door Forced

Door Held

Tamper/Security Alarms

Device Status Events

The source identifies broader OpenAccess event categories including access, door state, occupancy, alarm, device status, schedule, threat/security-mode, and personnel/credential events.

FR-EVT-003 – Event Normalization

OpenAccess events shall be transformed into a normalized Niagara event representation.

Recommended common fields:

Timestamp

Event Type

Event Category

Severity

Source ID

Source Name

Panel ID

Reader ID

Door/Logical Door

Description

FR-EVT-004 – Object Correlation

The driver shall correlate events with configured Niagara objects using stable identifiers provided by OnGuard.

FR-EVT-005 – Event Filtering

Events relating to panels/readers not configured in Niagara shall not update configured Niagara objects.

Your existing requirement says events should be matched against the locally selected panel/reader database.

FR-EVT-006 – Dynamic Event Configuration

The driver design shall allow additional supported event types to be introduced without redesigning the core event-processing architecture.

11. Niagara Alarm Requirements

FR-ALM-001 – Alarm Generation

Selected OpenAccess security/status events shall be convertible to Niagara alarms.

FR-ALM-002 – Alarm Source

Alarms shall be associated with the relevant Niagara component wherever sufficient source identification is available.

For example:

Communication Lost → Panel

Reader Offline → Reader

Door Forced → Logical Door

Door Held → Logical Door

FR-ALM-003 – Alarm Normalization

When the corresponding restoration event is received, the driver shall normalize the applicable Niagara alarm.

FR-ALM-004 – Alarm History

Alarm activation and restoration shall be retained through the applicable Niagara alarm/history mechanisms.

12. Access Event Requirements

FR-ACC-001 – Access Granted

The driver shall process approved Access Granted events received through OpenAccess event subscription.

FR-ACC-002 – Access Denied

The driver shall process approved Access Denied events.

FR-ACC-003 – Reader Correlation

Access events shall be correlated with the reader at which the event occurred where the required identifier is available.

Your current design specifically requires reader badge activity and alarms to be traceable to the corresponding reader.

FR-ACC-004 – Event History

Selected access events may be stored as Niagara histories or transient event records according to project configuration.

13. Synchronization and Recovery Requirements

FR-SYNC-001 – Startup Synchronization

After Niagara driver/station startup, the driver shall establish the current OnGuard state rather than waiting for future events.

Recommended sequence:

Driver Start

↓

Determine API Compatibility

↓

Authenticate

↓

Validate Configured Panels

↓

Retrieve Panel Status

↓

Retrieve Reader Status where supported

↓

Rebuild/Validate Logical Door Associations

↓

Establish Event Subscription

↓

ONLINE

FR-SYNC-002 – Reconnection Synchronization

Following a prolonged communication failure, the driver shall reconcile current status with OnGuard.

FR-SYNC-003 – Event Gap Handling

The driver shall not assume that the last locally known state remains valid following a communication/subscription outage.

FR-SYNC-004 – Restart Recovery

After Niagara restart, configured device states shall be initialized from current available OnGuard information rather than waiting indefinitely for the next status-change event.

14. Niagara Object Model

The proposed Niagara hierarchy shall be:

Drivers

│

└── LenelOnGuardNetwork

│

├── Connection Status

├── Authentication Status

├── Session Status

├── API Version/Profile

├── Event Subscription Status

├── Diagnostics

│

└── Devices

│

├── Panel-01

│   │

│   ├── Panel ID

│   ├── Type

│   ├── Firmware

│   ├── Online

│   ├── Health

│   │

│   ├── Readers

│   │   │

│   │   ├── Reader-01

│   │   │   ├── Reader ID

│   │   │   ├── Name

│   │   │   ├── Status

│   │   │   ├── Mode

│   │   │   ├── Last Event

│   │   │   └── Last Event Time

│   │   │

│   │   └── Reader-02

│   │

│   └── Doors

│       │

│       └── MainEntrance

│           ├── Entry Reader Ref

│           ├── Exit Reader Ref

│           ├── Position

│           ├── Forced

│           ├── Held

│           └── Last Event

│

└── Panel-02

15. V1 In-Scope Functional Requirements

The following shall be considered V1 scope:

| ID | Feature | Scope |
| --- | --- | --- |
| SCOPE-01 | OpenAccess HTTPS connectivity | In Scope |
| SCOPE-02 | Integration account authentication | In Scope |
| SCOPE-03 | Session/token management | In Scope |
| SCOPE-04 | Reauthentication | In Scope |
| SCOPE-05 | Password-expiry monitoring/warning | In Scope |
| SCOPE-06 | API-version compatibility management | In Scope |
| SCOPE-07 | Panel/controller discovery | In Scope |
| SCOPE-08 | Panel online/offline status | In Scope |
| SCOPE-09 | Reader discovery | In Scope |
| SCOPE-10 | Reader hardware/status monitoring where supported | In Scope |
| SCOPE-11 | Read-only reader configuration | In Scope |
| SCOPE-12 | Logical/virtual door representation | In Scope |
| SCOPE-13 | Approved door/security state monitoring | In Scope |
| SCOPE-14 | Event subscription | In Scope |
| SCOPE-15 | Access Granted/Denied events | In Scope |
| SCOPE-16 | Communication Lost/Restored | In Scope |
| SCOPE-17 | Approved security alarms | In Scope |
| SCOPE-18 | Niagara alarm generation/normalization | In Scope |
| SCOPE-19 | Event/alarm history | In Scope |
| SCOPE-20 | Startup/reconnection synchronization | In Scope |
| SCOPE-21 | Driver diagnostics/health | In Scope |

16. V1 Out-of-Scope Functional Requirements

The following shall explicitly be considered out of scope for V1:

| ID | Feature | Scope |
| --- | --- | --- |
| OOS-01 | Create/modify/delete cardholders | Out |
| OOS-02 | Identity/person administration | Out |
| OOS-03 | Badge administration | Out |
| OOS-04 | Credential provisioning | Out |
| OOS-05 | Access-level administration | Out |
| OOS-06 | PIN administration | Out |
| OOS-07 | Door Unlock/Lock commands | Out |
| OOS-08 | Reader mode/state commands | Out |
| OOS-09 | OnGuard configuration write-back | Out |
| OOS-10 | Rename OnGuard devices from Niagara | Out |
| OOS-11 | Expired-password recovery/reset through Niagara | Out |
| OOS-12 | Password-policy modification | Out |
| OOS-13 | Threat/security-mode commands | Out |
| OOS-14 | Alarm acknowledgment write-back to OnGuard | Out |
| OOS-15 | Direct OnGuard SQL database access | Out |

The V1 source explicitly excludes cardholder administration, access-level changes, door commands and write-back.

17. Open Requirements / Vendor Clarifications

These items should be marked TBC/TBD in the functional specification until confirmed.

FR-TBC-001 – Reader Status

Lenel shall confirm the supported mechanism for retrieving initial reader online/offline status, including the semantics of the Get Hardware Status result.

FR-TBC-002 – Reader Status Events

Lenel shall identify the exact events representing:

Reader Online

Reader Offline

Reader Communication Lost

Reader Communication Restored

FR-TBC-003 – Door Object

Lenel shall confirm whether a separately discoverable access-control door object exists for the required OpenAccess API versions or whether Niagara shall construct the logical door entirely from reader configuration.

FR-TBC-004 – Door Events

Lenel shall identify the exact events and identifying attributes for:

Door Open

Door Closed

Door Forced

Door Held

Door Forced Restored

Door Held Restored

FR-TBC-005 – Stable Object Identification

Lenel shall confirm whether Panel ID and Reader ID remain stable following OnGuard restart, configuration modification, backup/restore and upgrade.

FR-TBC-006 – Event Subscription Recovery

Lenel shall confirm whether a new event subscription is required following:

Session-token expiration

Reauthentication

OpenAccess restart

Niagara restart

Temporary network interruption

FR-TBC-007 – Event Durability

Lenel shall confirm which subscribed event categories are durable/recoverable and which events may be lost during an integration outage.

FR-TBC-008 – Session Duration

Security/Lenel shall confirm the approved production session-token duration and renewal strategy. The current notes record an eight-hour default but leave the appropriate production duration unresolved.

FR-TBC-009 – Password Expiry

Lenel shall confirm the exact API behavior when:

Password is approaching expiration

Password has expired

Password changes while an active token exists

New password is entered into Niagara

FR-TBC-010 – API Compatibility Matrix

Lenel shall provide/confirm the supported API version for each required function across the targeted OnGuard releases because the current design establishes that API versions need to be evaluated individually rather than as one global OpenAccess version.

This format gives you a requirements baseline that can be directly split into epics/stories: Connectivity & Authentication → API Versioning → Panel Discovery/Status → Reader Discovery/Status → Door Modeling → Event Subscription → Alarm Management → Synchronization/Recovery.

Top of Form

Bottom of Form