For your V1 Niagara 5–Lenel OnGuard driver, I would keep the high-level architecture deliberately simpler than the earlier diagram. Your current design says Niagara is primarily a read-only consumer: OnGuard remains the system of record, while Niagara uses OpenAccess REST APIs and event subscriptions for visualization, alarms, histories, analytics, and operational use.

High-level architecture

┌──────────────────────────────────────────────────────────────┐

│                    LENEL S2 ONGUARD                          │

│                 System of Record / Source                    │

│                                                              │

│   Controllers / Panels                                       │

│   Readers                                                    │

│   Access & Security Events                                   │

│   Alarm / Communication Events                               │

│   Password / Session Policy                                  │

│                                                              │

│            ┌────────────────────────────┐                    │

│            │     OpenAccess Platform    │                    │

│            │                            │                    │

│            │ • Authentication API       │                    │

│            │ • Discovery / Status REST  │                    │

│            │ • Event Subscription       │                    │

│            │ • Password Policy API      │                    │

│            └─────────────┬──────────────┘                    │

└──────────────────────────┼───────────────────────────────────┘

│

HTTPS / REST / Events

│

▼

┌──────────────────────────────────────────────────────────────┐

│                     NIAGARA 5 STATION                        │

│                                                              │

│              Lenel OpenAccess Driver                         │

│                                                              │

│  ┌──────────────┐   ┌──────────────┐   ┌─────────────────┐ │

│  │Authentication│   │ Discovery &  │   │Event Subscription│ │

│  │& Session Mgr │   │ Status Mgr   │   │Manager           │ │

│  └──────┬───────┘   └──────┬───────┘   └────────┬────────┘ │

│         │                  │                     │           │

│         └──────────────────┼─────────────────────┘           │

│                            ▼                                 │

│                 Data Normalization Layer                     │

│                            │                                 │

│                            ▼                                 │

│                   Niagara Object Model                       │

│                                                              │

│     Network → Controllers → Readers / Virtual Doors          │

│                     │                                        │

│                     ├── Status                               │

│                     ├── Events                               │

│                     ├── Alarms                               │

│                     └── Histories                            │

└────────────────────────────┬─────────────────────────────────┘

│

▼

┌──────────────────────────────────────────────────────────────┐

│               NIAGARA SUPERVISOR / HMI                      │

│                                                              │

│   Device Status │ Security Events │ Alarms │ Driver Health   │

└──────────────────────────────────────────────────────────────┘

This matches the first-release direction in your document: HTTPS connectivity, dedicated least-privilege authentication, panel/controller/reader/door discovery, Niagara status objects, event subscriptions, and conversion of selected events to alarms/history. Your V1 specifically excludes cardholder administration, access-level changes, door commands and write-back.

What data should be displayed in Niagara?

I would divide the UI into five views, rather than exposing raw OpenAccess API data directly.

| Niagara View | Data to Display |
| --- | --- |
| 1. OnGuard Network | Connection Status, OnGuard Server, OpenAccess/API Status, OnGuard Version, API Version/Profile, Last Successful Communication, Last Sync, Event Subscription Status |
| 2. Authentication / Security | Authentication Status, Session Status, Token Expiry Time, Token Time Remaining, Password Expiry Status, Password Expiry Date/Days Remaining, Last Authentication, Certificate Status |
| 3. Controllers / Panels | Controller/Panel Name, ID, Type, Firmware, Online/Offline, Health, Last Successful Read, Last Status Change |
| 4. Readers / Doors | Reader Name/ID, Parent Controller, Online/Offline where supported, Reader Mode, Associated/Virtual Door, Door State where available, Last Event, Last Event Time |
| 5. Events & Alarms | Event Type, Timestamp, Source Name, Controller ID, Reader ID, Access Granted/Denied, Communication Lost/Restored, Door Forced/Held, Security Alarm, Alarm State/Acknowledgement |

Your source already proposes controller properties such as ID, name, type, firmware, online state, last successful read, last status change and health, while readers include ID, controller ID, name, online state, mode, strike time, entry area and exit area.

Recommended Niagara dashboard

For an operator, I would avoid showing every technical property on the main page. The top-level dashboard could be:

┌─────────────────────────────────────────────────────────────┐

│             LENEL ONGUARD INTEGRATION                       │

│                                                             │

│ Connection       Authentication      Event Subscription     │

│ ● ONLINE         ● AUTHENTICATED     ● ACTIVE               │

│                                                             │

│ OnGuard Version: 8.x        API Profile: xxxx               │

│ Last Sync:       10:42:15    Last Event: 10:42:51           │

├─────────────────────────────────────────────────────────────┤

│                    DEVICE HEALTH                            │

│                                                             │

│ Controllers     Readers          Offline Devices            │

│     12            148                   2                    │

├─────────────────────────────────────────────────────────────┤

│                  SECURITY STATUS                            │

│                                                             │

│ Active Alarms        Door Alarms       Communication Alarms │

│      4                    2                    2              │

├─────────────────────────────────────────────────────────────┤

│                 AUTHENTICATION HEALTH                       │

│                                                             │

│ Session: ACTIVE                                             │

│ Token Expires:       17:30                                  │

│ Token Remaining:     06h 42m                                │

│ Password Status:     VALID                                  │

│ Password Expires In: 23 days                                │

├─────────────────────────────────────────────────────────────┤

│                    RECENT EVENTS                            │

│                                                             │

│ Time      Source        Event                 State          │

│ 10:42:51  Reader-12     Access Granted        Normal         │

│ 10:41:32  Panel-03      Communication Lost    Alarm          │

│ 10:40:14  Door-21       Door Forced           Alarm          │

│ 10:39:51  Panel-02      Communication Restored Normal        │

└─────────────────────────────────────────────────────────────┘

Authentication data should be visible, but credentials should not

Since authentication is operationally important, expose state, not secrets:

Authentication Status     AUTHENTICATED

Session Status            ACTIVE

Last Authentication       28-Sep-2026 10:00

Token Expiration          28-Sep-2026 18:00

Token Remaining           07:15:32

Password Status           VALID

Password Expires          21-Oct-2026

Password Remaining        23 Days

Password Warning          FALSE

Last Auth Failure         -

Reauthentication Count    1

Do not display:

Password

Session Token

Authentication Cookie

Credential payload

Your document specifically separates token expiration from password expiration and requires the session manager to capture and manage those independently.

Controller view

When the operator drills down:

OnGuard Network

│

└── Controllers

│

├── Panel-01

│      Status          ONLINE

│      Panel ID        1001

│      Type            LNL-xxxx

│      Firmware        x.x

│      Health          NORMAL

│      Last Update     10:42:10

│

├── Panel-02

│      Status          ONLINE

│

└── Panel-03

Status          OFFLINE

Alarm           COMMUNICATION LOST

For your implementation, communication lost → Offline + Niagara alarm, and communication restored → Online + alarm normalization, with the event retained in alarm history. That behavior is explicitly captured in your design notes.

Reader / door view

I would model the UI approximately as:

Panel-01

│

├── Reader-01

│      Reader ID

│      Status

│      Mode

│      Associated Door

│      Last Event

│

├── Reader-02

│

└── Virtual Doors

│

├── Main Entrance

└── Service Entrance

There is an important implementation constraint here: your latest design discussion says OnGuard does not appear to expose a separate physical door type for this flow; doors are effectively virtual/logical groupings associated with readers. The current decision is therefore reader discovery with virtual door groupings rather than treating every door as an independently discovered hardware device.

Event display

For event records, normalize everything into one Niagara representation:

Timestamp

Event Type

Event Category

Severity

Source Name

Source ID

Panel ID

Reader ID

Door/Location

Description

Alarm State

Then specific events can add relevant information:

ACCESS GRANTED

├── Reader

├── Door/Location

├── Timestamp

└── Approved person information (if permitted)

ACCESS DENIED

├── Reader

├── Door/Location

├── Timestamp

└── Denial reason (if available)

COMMUNICATION LOST

├── Panel/Device ID

├── Device Name

├── Timestamp

└── Alarm = Active

COMMUNICATION RESTORED

├── Panel/Device ID

├── Timestamp

└── Alarm = Normalized

The source identifies access granted/denied, door-state, occupancy, alarm, device-status, schedule, threat/security-mode, and personnel/credential events as available event categories, although the exact V1 subset still needs to follow your approved scope.

So for V1, the cleanest architecture is: OnGuard → OpenAccess → Niagara Driver → normalized Controllers/Readers/Virtual Doors → Status + Events + Alarms + History → Niagara Supervisor/HMI. Keep cardholder administration and write-back out of the architecture, and make connection health, authentication/session health, controller/reader health, security events and alarms the primary information visible to operators.

Top of Form

Bottom of Form

Based on the design in your document, I would model the Niagara 4/5 driver hierarchy around the physical OnGuard panel/controller first, with readers beneath the panel and a logical/virtual door layer associating the appropriate reader(s). This fits your latest discovery finding that panels are directly discoverable, while doors are not exposed as an independent physical device type in the same way and therefore need logical grouping.

1. Recommended Niagara component hierarchy

Drivers

│

└── LenelOnGuardNetwork

│

├── Network Configuration

│

├── Authentication / Session

│

├── Event Subscription

│

├── Diagnostics

│

└── Devices

│

├── Panel-01

│   │

│   ├── Panel Status

│   │

│   ├── Readers

│   │   │

│   │   ├── Reader-01

│   │   ├── Reader-02

│   │   └── Reader-03

│   │

│   ├── Doors

│   │   │

│   │   ├── Main-Entrance

│   │   └── Service-Entrance

│   │

│   └── Events / Alarms

│

├── Panel-02

│   ├── Readers

│   ├── Doors

│   └── Events / Alarms

│

└── Panel-03

├── Readers

├── Doors

└── Events / Alarms

At the Java/component level:

BLenelNetwork

│

└── BLenelDevice              ← Panel / Controller

│

├── BLenelPanelStatus

│

├── BLenelReader

│

├── BLenelReader

│

├── BLenelDoor         ← logical/virtual object

│

└── BLenelEventContainer

Your source already defines BLenelDevice as the panel/controller and proposes BLenelReader, BLenelDoor, BLenelAccessDoor, and a read-only point extension.

2. Panel / Controller structure

The panel should be the primary Niagara device.

BLenelDevice / BLenelPanel

│

├── Identification

│   ├── panelId

│   ├── name

│   ├── type

│   ├── firmware

│   └── communicationServerId

│

├── Status

│   ├── online

│   ├── health

│   ├── lastSuccessfulRead

│   ├── lastStatusChange

│   └── lastCommunication

│

├── Readers

│   ├── Reader-01

│   ├── Reader-02

│   └── Reader-N

│

├── Doors

│   ├── Door-01

│   └── Door-N

│

└── Events

├── CommunicationLost

├── CommunicationRestored

└── SystemEvents

Panel properties

Your current requirements directly support these properties:

| Property | Niagara Type | Access | Purpose |
| --- | --- | --- | --- |
| panelId | String | Read Only | Unique OnGuard panel identifier |
| name | String | Read Only | OnGuard configured name |
| type | String/Enum | Read Only | Panel/controller type |
| firmware | String | Read Only | Firmware/version |
| online | BooleanStatus | Read Only | Online/offline |
| health | Enum/String | Read Only | Normal/fault/unknown |
| lastSuccessfulRead | AbsTime | Read Only | Last successful REST/status read |
| lastStatusChange | AbsTime | Read Only | Last status transition |

These align directly with the controller model already captured in your design.

I would additionally maintain internal driver metadata such as:

apiObjectId

lastPollTime

lastEventTime

statusSource

discoveryState

These are implementation recommendations rather than confirmed OpenAccess fields.

3. Panel status flow

Initial state should come from discovery/status retrieval.

OpenAccess REST API

│

▼

Discover Panel

│

▼

Get Panel Properties

│

▼

BLenelDevice

│

├── panelId

├── name

├── firmware

└── online

After initialization, events maintain the state:

COMMUNICATION LOST

│

▼

Event Subscription

│

▼

Event Parser

│

▼

Find Panel by ID

│

▼

panel.online = false

│

├── Update Niagara point

├── Raise alarm

└── Add history

And:

COMMUNICATION RESTORED

│

▼

panel.online = true

│

├── Update point

├── Normalize alarm

└── Record history

That communication-lost/restored behavior is specifically part of your current design.

4. Reader structure

Each discovered reader should be associated with its parent panel.

Panel-01

│

└── Readers

│

├── Reader-01

│   │

│   ├── Configuration

│   │   ├── readerId

│   │   ├── name

│   │   ├── controllerId

│   │   ├── mode

│   │   ├── strikeTime

│   │   ├── entryArea

│   │   └── exitArea

│   │

│   ├── Status

│   │   ├── online

│   │   ├── lastStatusChange

│   │   └── lastEvent

│   │

│   └── Events

│       ├── AccessGranted

│       ├── AccessDenied

│       └── ReaderAlarm

│

└── Reader-02

Your current reader model explicitly identifies readerId, controllerId, name, online, mode, strikeTime, entryArea, and exitArea.

5. Important issue with Reader online

I would be careful with this point.

Your API investigation found that reader details do not directly provide an online/offline field in the same way as panel status. The team identified a separate hardware-status mechanism and reader connectivity/status events as possible mechanisms.

Therefore don't simply implement:

Reader API

↓

online=true/false

until the API behavior is confirmed.

Instead:

Reader Status

▲

│

┌───────────┴──────────┐

│                      │

Initial status             Runtime status

│                      │

Get Hardware Status       Event Subscription

│                      │

└───────────┬──────────┘

▼

BLenelReader.online

And if the API doesn't provide a reliable reader status:

online = UNKNOWN

is preferable to inferring it from unrelated fields. Your notes explicitly record that decision principle.

6. Reader mode

Reader mode should be a Niagara read-only enum rather than an arbitrary integer exposed to operators.

For example:

BLenelReader

│

└── readerMode

│

├── CardOnly

├── PINOnly

├── CardOrPIN

├── CardAndPIN

├── Locked

├── Unlocked

└── Unknown

However, those exact enum values should be mapped against the actual Lenel reader-mode table for the target API release rather than assumed.

Your source specifically notes that offline mode 2 is a reader-mode setting used when communication between the reader and panel is lost, and that the Lenel reader-mode table should be the reference for interpreting it.

7. Door structure

This needs a different design from Panel and Reader.

Your investigation concluded that the API did not expose the door as a separate physical device type in the examined flow. Instead, the proposed door is a logical/virtual grouping associated with reader configuration.

So I would model:

BLenelDoor

│

├── Identity

│   ├── doorId / logicalId

│   ├── displayName

│   └── panelId

│

├── Association

│   ├── entryReaderId

│   └── exitReaderId

│

├── State

│   ├── doorState

│   ├── forced

│   ├── held

│   ├── communicationStatus

│   └── lastEvent

│

└── Alarm Extensions

├── forcedAlarm

└── heldAlarm

Conceptually:

Panel-01

│

┌─────────────┴─────────────┐

│                           │

Reader-01                   Reader-02

Entry Reader                Exit Reader

│                           │

└─────────────┬─────────────┘

│

▼

Main Entrance

[Virtual Door]

│

┌────────┴────────┐

│                 │

Door Forced       Door Held

│                 │

▼                 ▼

Niagara Alarm    Niagara Alarm

8. Don't duplicate readers underneath doors

I would avoid this physical Niagara hierarchy:

Panel

└── Door

├── EntryReader

└── ExitReader

because the reader is the discoverable OnGuard hardware entity, while the door is the logical association.

Instead:

Panel

│

├── Readers

│    ├── Reader-01

│    └── Reader-02

│

└── Doors

└── MainEntrance

entryReaderRef → Reader-01

exitReaderRef  → Reader-02

This prevents duplicate reader objects.

9. Door state

I would expose a derived enum:

doorState

NORMAL

OPEN

CLOSED

FORCED

HELD

UNKNOWN

but distinguish physical position from alarm/event condition internally.

For example:

BLenelDoor

│

├── position

│      OPEN

│      CLOSED

│      UNKNOWN

│

├── forced

│      true / false

│

├── held

│      true / false

│

└── derivedDoorState

NORMAL

OPEN

CLOSED

FORCED

HELD

UNKNOWN

That is more robust than making everything one boolean.

Your existing door proposal already contains communicationStatus, derivedDoorState, associatedReaderId, and lastEvent.

10. Events should update existing objects

Avoid creating a permanent Niagara component for every event.

Instead:

OpenAccess Event

│

▼

EventSubscriber

│

▼

EventNormalizer

│

▼

Extract:

panelId

readerId

eventType

source

timestamp

│

▼

Object Registry

│

├── Find Panel

├── Find Reader

└── Find Logical Door

│

▼

Update properties

│

┌───────┼─────────┐

▼       ▼         ▼

Point    Alarm     History

For example:

AccessDenied

PanelId  = 100

ReaderId = 2001

↓

Panel-01

└── Reader-01

│

├── lastEvent = ACCESS_DENIED

├── lastEventTime = ...

└── Event History

Your requirement specifically says reader badge activity and alarms should appear against the corresponding reader so an access-denied event can be traced back to its reader.

11. Discovery structure

The Workbench discovery flow should ideally be:

Lenel Network

│

▼

Discover

│

▼

OpenAccess

│

▼

Available Panels

│

├── Panel-01

├── Panel-02

└── Panel-03

User selects:

☑ Panel-01

☑ Panel-02

☐ Panel-03

│

▼

Add

│

▼

Discover Readers

for selected panels

│

▼

Panel-01

├── Reader-01

├── Reader-02

└── Reader-03

That matches your requirement that users should select panels and then add only readers discovered under those selected panels. Events for hardware that hasn't been added to Niagara should not be displayed/processed as configured Niagara objects.

12. Final Niagara 4 station representation

I would target something close to:

Drivers

│

└── LenelOnGuardNetwork

│

├── connectionStatus

├── serviceHealth

├── server

├── apiVersion

├── authenticationStatus

├── sessionStatus

├── eventSubscriptionStatus

│

└── Devices

│

├── Panel_01

│   │

│   ├── panelId

│   ├── panelType

│   ├── firmware

│   ├── online

│   ├── health

│   │

│   ├── Readers

│   │   │

│   │   ├── Reader_01

│   │   │   ├── readerId

│   │   │   ├── name

│   │   │   ├── online

│   │   │   ├── mode

│   │   │   ├── entryArea

│   │   │   ├── exitArea

│   │   │   ├── lastEvent

│   │   │   └── lastEventTime

│   │   │

│   │   └── Reader_02

│   │

│   └── Doors

│       │

│       └── MainEntrance

│           ├── entryReaderRef → Reader_01

│           ├── exitReaderRef  → Reader_02

│           ├── position

│           ├── forced

│           ├── held

│           ├── derivedDoorState

│           └── lastEvent

│

└── Panel_02

├── Readers

└── Doors

The important design distinction is therefore Panel = Niagara device, Reader = discoverable child hardware component, Door = logical/virtual component referencing one or more readers. Status should initialize from the appropriate REST/hardware-status mechanism and subsequently be maintained through OpenAccess events; all discovered OnGuard configuration properties should remain read-only in V1, consistent with your current project decisions