For the Inventory & Identifiers requirement, there are two levels of detail:

- What your RFI already confirms about the Lenel OpenAccess deployment.

- The exact object/resource operations Lenel must provide for your installed OnGuard/OpenAccess version.

Your RFI gives the OpenAccess client endpoint pattern approximately as:

https://<FQDN>:port/api/access/onguard/openaccess

and mentions OpenAccess classes such as Panel, Reader, AlarmInput, Area. However, the RFI itself does not contain the final resource paths, query syntax, or response property names; in fact Section 10 explicitly asks Lenel to provide that mapping. So I would not treat paths such as /panels, /readers, etc. as confirmed Lenel endpoints unless they appear in the SDK/OpenAccess documentation Lenel supplies.

2. Mandatory baseline requirements

Live Status and Alarms, the requirement is that your iBMS should not only know what Lenel objects exist, but should continuously understand their current operational state and react to changes in near real time.

The main categories are:

Device / Controller -  Online/Offline [This applies to things such as controllers, readers, communication servers, and possibly panels]

Communication – Lost/Restored

Reader / Door – Forced/Held/Tamper

[A Forced Door condition usually means the door became open without an authorized access sequence.]

[Held Door means the door remained open longer than the configured allowed-open duration]

Power / Voltage  - Power failure/Low voltage/Power restored

[Specifically calls out power and voltage-related states]

Alarm – Active/Restored

[Life cycle of alarm status]

The important design point is that this is usually event-driven

Event Feed

Used for: - Access Granted - Access Denied - PIN events - Emergency events - Area Control events - alarms - status changes - recovery/restored events

Events you need to subscribe to above events.

Access Denied can be one of the below reasons:

Examples include: Invalid badge, Invalid PIN, Time schedule restriction

Area Control includes operational events such as: Area occupancy limit exceeded, Area closed

For the Door Control / Unlock Door requirement, the migration must preserve the existing operator ability to remotely unlock an authorized door through Lenel OnGuard, but the command should flow through your backend and be fully controlled and auditable.

Current reader state/mode -> SetMode command ->New reader operating mode

Reader operating mode can be Normal/Unlocked /  Card Only / PIN Only

Cardholder Provisioning | +-- Create Cardholder +-- Create Badge +-- Assign Access Level +-- Manage PIN +-- Disable Credential

OpenAccess Services

Client Endpoint

This is the endpoint that our application will access.

The document gives a pattern approximately like:

<protocol>://<host>:8080/api/access/onguard/openaccess

LS OpenAccess Service:

This is the main OpenAccess service.

Lenel needs to confirm:

Host

Service account

Startup configuration

Recovery configuration

System Options assignment

Windows Server: The service account is the Windows/service-side account running OpenAccess

LS Web Service / NGINX:

Listening port TLS configuration Reverse proxy Load balancer

LS Web Event Bridge

This component is mainly for real-time events.

The document explicitly mentions: SignalR / WebSocket, ConnectionHeartbeat (~10 seconds), Re-subscribe after restart

So your Java event architecture should be approximately:

Some events could be   +--> Access Granted

+--> Access Denied

+--> Door Forced

+--> Door Held

+--> Comm Lost

+--> Comm Restored

+--> Alarm

Authentication

The first item is the integration account.

The document says OpenAccess authenticates through:

Internal account

or

Normal SSO

Lenel needs to confirm which mechanism MDD will use.

For example:

MDD Java

|

| integration credentials

v

OpenAccess

|

v

Authentication

The RFI asks for:

MDD iBMS account

Directory ID if applicable

Credential rotation process

27. Authentication/session model

The document references:

Token

Session

Cookie

and says the documented/default OpenAccess idle session is approximately:

8 hours

but Lenel must confirm the actual configuration.

Your Java application therefore needs something like:

LenelSessionManager

|

+-- Authenticate

|

+-- Store session/cookie

|

+-- Detect expiration

|

+-- Reauthenticate

|

+-- Reconnect event subscription

Don't assume OAuth2/Bearer token unless the actual OpenAccess documentation confirms that mechanism.