The architecture is best understood as two layers: the overall iBMS integration architecture, and inside Niagara, a custom Niagara 5 Lenel OpenAccess driver responsible for discovery, real-time events, status, commands, session handling, and conversion of Lenel objects into Niagara components/points.

1. High-level architecture

The primary data path is:

Lenel S2 OnGuard

│

│ Internal OnGuard services

▼

OpenAccess REST API

│

├──── REST/HTTPS ──────► Inventory / Status / Commands

│

└──── Event Channel ───► Real-time Events

│

▼

Niagara 5 OpenAccess Driver

│

┌─────────────┼──────────────┐

▼             ▼              ▼

Points         Alarms         History

│             │              │

└─────────────┼──────────────┘

▼

Niagara Station

│

┌────────────┴────────────┐

▼                         ▼

Niagara Supervisor          Other iBMS

Workbench / HMI          HVAC / Fire / etc.

Lenel side

The Lenel environment remains the system of record for access control. Niagara should not communicate directly with the OnGuard SQL database.

The integration boundary should be:

Niagara

│

▼

OpenAccess

│

▼

OnGuard Services

│

▼

OnGuard Database

This separation is important because the supplied RFI explicitly identifies OpenAccess as the supported integration interface and states that direct database access is restricted.

The important Lenel components are OnGuard Application/Services, OpenAccess REST API, LS Event Context Provider, LS Message Broker / Web Event Bridge where applicable, and the OnGuard SQL Server behind the application layer.

The Event Context Provider being on the same host as OpenAccess is particularly important for the event architecture.

2. Two integration paths

The Niagara driver should not treat everything as polling. There are two fundamentally different paths.

REST/API path

Used for operations such as:

Authentication

Asset discovery

Initial status retrieval

Configuration

Door command

Reader mode command*

Threat/security mode command*

Recovery/resynchronization

Conceptually:

Niagara Driver

│

│ HTTPS 443

▼

OpenAccess REST API

│

▼

OnGuard

The driver sends JSON requests and converts the returned OpenAccess objects into Niagara objects.

Real-time event path

Events should use the OpenAccess event mechanism supported by the deployed release rather than repeatedly calling REST endpoints.

OnGuard event occurs

│

▼

Event Context Provider

│

▼

OpenAccess Event Service

│

│ WebSocket/Event subscription

▼

Niagara EventSubscriber

│

▼

Event Parser

│

▼

Point / Alarm / History

For example:

Door forced open

↓

OnGuard

↓

OpenAccess Event

↓

Niagara EventSubscriber

↓

Object mapping

↓

Door-01.forced = true

↓

Niagara Alarm Service

This gives Niagara near-real-time updates without aggressive polling.

3. Recommended Niagara low-level driver design

At Niagara-driver level, I would structure the module approximately like this:

BLenelNetwork

│

├── BLenelConfig

│

├── BLenelSessionManager

│

├── BLenelHttpClient

│

├── BLenelDiscoveryManager

│

├── BLenelEventManager

│

├── BLenelCommandManager

│

├── BLenelHealthManager

│

├── BLenelMappingManager

│

└── Device/Object Components

│

├── BLenelPanel

│    └── BLenelReader

│          └── BLenelDoor

│

├── BLenelArea

├── BLenelInput

├── BLenelOutput

└── BLenelAlarmPoint

The exact Baja classes/interfaces depend on the Niagara 5 driver SDK/framework available for your build, but this separation of responsibilities is the important design principle.

4. BLenelNetwork

This should be the root Niagara component representing the OpenAccess connection.

For example:

Drivers

│

└── LenelOpenAccessNetwork

│

├── Config

├── Health

├── Panels

├── Readers

├── Doors

├── Areas

├── Inputs

└── Outputs

Suggested properties:

serverUrl

port

applicationId

username

credentialReference

connectionState

sessionState

lastConnected

lastEventReceived

lastSync

eventSubscriptionState

reconnectCount

apiFailureCount

Do not expose passwords/tokens as ordinary Niagara string properties. They should use Niagara's secure credential/password mechanisms.

5. Session Manager

The RFI is particularly important here because it refers to session/token/cookie handling and an approximately eight-hour idle/session behavior that still requires site/version confirmation.

Therefore isolate authentication into:

BLenelSessionManager

Responsibilities:

authenticate()

getSession()

validateSession()

refreshSession()

invalidateSession()

reauthenticate()

logout()

The API client should never independently manage authentication.

Instead:

API Client

│

▼

SessionManager.getValidSession()

│

├── valid → continue

│

└── invalid

│

▼

authenticate

│

▼

return session

This becomes especially important after network outages or OpenAccess restart.

6. API client

Create a centralized API abstraction:

LenelOpenAccessClient

Internally:

LenelOpenAccessClient

│

├── AssetApi

├── DoorApi

├── ReaderApi

├── ControllerApi

├── AlarmApi

├── AreaApi

├── InputOutputApi

└── CommandApi

For example:

interface DoorApi {

List<DoorDto> getDoors();

DoorStatusDto getDoorStatus(String id);

CommandResult unlock(String id);

}

The important point is that your Niagara components should not construct HTTP calls themselves.

Bad:

BLenelDoor

↓

HTTP call

Preferred:

BLenelDoor

↓

CommandManager

↓

OpenAccessClient

↓

HTTP

That makes authentication, retries, logging and version changes much easier to control.

7. Discovery architecture

The RFI requires inventory/asset discovery.

I would implement:

BLenelDiscoveryManager

which discovers:

Controllers/Panels

Readers

Doors

Alarm points

Inputs

Outputs

Areas

Segments

Locations

Communication servers

Processing:

GET OpenAccess Assets

│

▼

JSON response

│

▼

DTO / Model parser

│

▼

Lenel object ID

│

▼

Niagara component mapping

│

▼

Niagara database

A critical design rule is to retain the Lenel object ID.

For example:

BLenelDoor

name             = "Main Lobby Door"

lenelObjectId    = "48372"

panelId          = "1004"

readerId         = "23891"

location         = "Ground Floor"

Do not use the door name as the integration key because names can change.

8. Object registry

One component that I would add to the diagram is a central registry:

LenelObjectRegistry

For example:

ConcurrentMap<String, BComponent>

48372 → MainLobbyDoor

48373 → ServiceEntrance

19201 → LobbyReader

1004  → Controller01

When an event arrives:

Event

objectId = 48372

│

▼

ObjectRegistry

│

▼

MainLobbyDoor

│

▼

Update Niagara points

Without this registry, every event may require searching the Niagara component tree, which is unnecessary and inefficient.

9. Niagara object model

A door could expose something like:

BLenelDoor

│

├── online

├── doorPosition

├── lockState

├── forced

├── held

├── alarm

├── lastEvent

├── lastEventTime

│

└── Actions

├── unlock

└── lock*

A reader:

BLenelReader

│

├── online

├── communicationStatus

├── readerMode

├── tamper

├── lastCardEvent

└── lastEventTime

Controller:

BLenelController

│

├── online

├── communicationStatus

├── powerStatus

├── batteryStatus

├── tamper

└── lastCommunication

The final points depend on what the installed OpenAccess version actually exposes.

10. Event manager

This is one of the most important classes.

BLenelEventManager

Internally:

EventManager

│

├── SubscriptionManager

├── WebSocketClient

├── EventParser

├── EventFilter

├── EventRouter

└── ReconnectManager

Processing pipeline:

OpenAccess

│

▼

WebSocket/Event API

│

▼

Raw JSON

│

▼

EventParser

│

▼

LenelEvent

│

▼

EventFilter

│

▼

EventRouter

│

├── Door Event

├── Reader Event

├── Access Event

├── Alarm Event

└── System Event

For example:

ACCESS_GRANTED

ACCESS_DENIED

DOOR_FORCED

DOOR_HELD

DOOR_RESTORED

COMM_LOST

COMM_RESTORED

TAMPER

POWER_FAILURE

SYSTEM_RESET

11. Event-to-Niagara mapping

Don't hard-code event processing throughout individual components.

Use a mapping layer:

Lenel Event             Niagara behavior

--------------------------------------------------

Door Forced       →     forced = true + alarm

Door Restored     →     forced = false + alarm clear

Door Held         →     held = true + alarm

Comm Lost         →     online = false + alarm

Comm Restored     →     online = true

Access Granted    →     event/history

Access Denied     →     event/history/alarm*

This mapping can sit in:

BLenelMappingManager

That gives you one place to accommodate differences between OnGuard/OpenAccess versions.

12. Command architecture

Commands require stricter handling than status retrieval.

Use:

Niagara Action

│

▼

CommandManager

│

├── authorization

├── validation

├── audit

├── API execution

└── response processing

│

▼

OpenAccess API

For example:

Operator

↓

Door-01.unlock()

↓

Niagara permissions

↓

CommandManager

↓

validate door

↓

OpenAccessClient

↓

Unlock command

↓

OnGuard

↓

Result

↓

Niagara audit/history

This is preferable to exposing arbitrary REST command execution.

For your current scope, Unlock Door is the clear legacy-equivalent baseline command. SetMode, threat-level/security-mode commands, and command-issued alarm acknowledgements should remain separate optional/approved capabilities as reflected in your RFI.

13. Polling + event hybrid model

Even with event subscription, I would not make the driver 100% event dependent.

Use a hybrid model:

OpenAccess

/          \

/            \

REST polling       Events

│                │

▼                ▼

Reconciliation     Real-time update

│                │

└───────┬────────┘

▼

Niagara points

Events provide fast updates.

Periodic reconciliation provides recovery from:

Missed events

WebSocket disconnect

OpenAccess restart

Niagara restart

Network outage

Subscription failure

For example:

Real-time:

WebSocket events

Every X minutes:

lightweight health/status reconciliation

After reconnect:

forced full resynchronization

The exact polling intervals should be configurable rather than hard-coded.

14. Connection state machine

The driver should have an explicit state machine:

DISCONNECTED

│

▼

CONNECTING

│

▼

AUTHENTICATING

│

▼

DISCOVERING / SYNCING

│

▼

SUBSCRIBING

│

▼

ONLINE

│

├──────────────┐

│              │

session expired   connection lost

│              │

▼              ▼

REAUTHENTICATING   RECONNECTING

│              │

└──────┬───────┘

▼

ONLINE

This will make Niagara driver diagnostics much easier.

15. Threading design

Do not perform OpenAccess calls on Niagara's main component execution thread.

A reasonable internal model is:

Niagara Component Thread

│

▼

Work Queue

│

┌──────┴──────┐

▼             ▼

API Executor   Event Executor

│             │

▼             ▼

REST           WebSocket

Then marshal resulting component updates safely back through the Niagara framework.

Also avoid creating one thread per door/reader. One integration may have thousands of access-control objects.

16. Driver package structure

A clean Java module structure could look like:

com.company.niagara.lenel

│

├── network

│   ├── BLenelNetwork

│   └── LenelNetworkWorker

│

├── auth

│   ├── SessionManager

│   └── CredentialManager

│

├── api

│   ├── OpenAccessClient

│   ├── AssetApi

│   ├── DoorApi

│   └── CommandApi

│

├── event

│   ├── EventSubscriber

│   ├── EventParser

│   ├── EventRouter

│   └── ReconnectManager

│

├── discovery

│   ├── DiscoveryManager

│   └── ObjectRegistry

│

├── component

│   ├── BLenelController

│   ├── BLenelReader

│   ├── BLenelDoor

│   ├── BLenelArea

│   ├── BLenelInput

│   └── BLenelOutput

│

├── command

│   └── CommandManager

│

├── mapping

│   ├── EventMapper

│   └── StatusMapper

│

├── model

│   ├── DoorDto

│   ├── ReaderDto

│   └── LenelEvent

│

└── health

└── DriverHealthManager

17. Full low-level runtime flow

Putting all of it together:

NIAGARA 5 STATION

┌──────────────────────────────────────────────────────────────┐

│                                                              │

│ BLenelNetwork                                                │

│      │                                                       │

│      ├──── SessionManager ───────────────┐                   │

│      │                                   │                   │

│      ├──── DiscoveryManager              │                   │

│      │       │                           │                   │

│      │       └── ObjectRegistry          │                   │

│      │                                   ▼                   │

│      ├──── OpenAccessClient ─────────► HTTPS REST            │

│      │                                                       │

│      ├──── EventManager ─────────────► WebSocket/Event API   │

│      │       │                                               │

│      │       ├── Parser                                      │

│      │       ├── Filter                                      │

│      │       └── Router                                      │

│      │                                                       │

│      ├──── CommandManager ───────────► OpenAccess Commands   │

│      │                                                       │

│      └──── MappingManager                                    │

│              │                                               │

│              ▼                                               │

│      Niagara Component Model                                 │

│                                                              │

│ Controllers → Readers → Doors                                │

│ Areas                                                        │

│ Inputs / Outputs                                             │

│ Alarm Points                                                 │

│                                                              │

│        │              │               │                      │

│        ▼              ▼               ▼                      │

│      Points         Alarms          History                   │

│                                                              │

└──────────────────────────────────────────────────────────────┘

│

│ HTTPS 443 / approved event channel

▼

┌──────────────────────────────────────────────────────────────┐

│                    LENEL ONGUARD                             │

│                                                              │

│ OpenAccess REST API                                          │

│ Event Context Provider                                       │

│ Message Broker / Event Services                              │

│ OnGuard Services                                             │

│ OnGuard DB                                                   │

└──────────────────────────────────────────────────────────────┘

One design change I recommend to the current architecture

The current diagram is good at the solution level, but for an implementation design I would expand the Lenel OpenAccess Driver into these eight explicit blocks:

Lenel OpenAccess Driver

┌───────────────────────────────────────────────────┐

│                                                   │

│ Session Manager          Connection State Manager │

│                                                   │

│ OpenAccess REST Client   Discovery Manager        │

│                                                   │

│ Event Subscriber         Object Registry          │

│                                                   │

│ Command Manager          Mapping/Normalization    │

│                                                   │

│           Health / Diagnostics / Audit            │

└───────────────────────────────────────────────────┘

│

▼

Niagara Components

Controller / Reader / Door / Area / I/O

That decomposition will translate much more naturally into the actual Niagara 5 Java driver implementation, unit/integration tests, reconnect handling, and Lenel API mapping.

The key principle is: OpenAccess API objects should not become Niagara points directly. The driver should have an API/DTO layer → mapping layer → Niagara component model. That separation will protect the Niagara implementation if Lenel changes endpoints, JSON fields, event structures, or API versions.

Top of Form

Bottom of Form